Inhaltsverzeichnis

1  Zweck und Abgrenzung zum Lastenheft	3

2  Traceability: Lastenheft-Code → SCL / FUP	3

2.1  Allgemein / Querschnitt	3

2.2  Vision (AF-10…14)	3

2.3  RFID (AF-20…25)	3

2.4  Hochregallager (AF-30…38)	4

2.5  Montage, Förderer, HMI, Sicherheit	4

3  Sprachen: SCL und FUP	4

4  FUP-Codes (OB1-Netzwerke NW 1–29)	4

4.1  Stationen — Übersicht SCL / FUP / Tags	5

4.2  Stationen — alle Factory-I/O- und SPS-Tags	6

Station 1A — NW 1–2 — 6 Tags (FIO 6 / SPS 0)	6

Station 1B — NW 4, 6 — 6 Tags (FIO 6 / SPS 0)	6

Station 2A — NW 3, 5 — 31 Tags (FIO 12 / SPS 19)	6

Station 2B — NW 7–8 — 31 Tags (FIO 13 / SPS 18)	7

Station 2A/2B Band — NW 3 / 8 / 26 / 27 — 9 Tags (FIO 4 / SPS 5)	8

Station 3A — NW 9–12 — 33 Tags (FIO 32 / SPS 1)	9

Station 3B — NW 17–20 — 36 Tags (FIO 35 / SPS 1)	10

Station 4A — NW 13–16, 21 — 83 Tags (FIO 30 / SPS 53)	11

Station 4B — NW 22–25 — 29 Tags (FIO 29 / SPS 0)	13

Station 5A — NW 26, 28 Warehouse_2 — 108 Tags (FIO 35 / SPS 73)	14

Station 5B — NW 27, 29 Warehouse_1 — 111 Tags (FIO 33 / SPS 78)	17

Station Standard-Variablentabelle — — — 2 Tags (FIO 1 / SPS 1)	20

5  SCL-Codes (Quelldateien im Repository)	20

5.1  Vision, RFID, Handling	20

5.2  Warehouse_1 Kunststoff (FUP NW 29)	20

5.3  Warehouse_2 Metall (FUP NW 28)	21

6  Produktcodes	21

7  Lager-Info_Code 0–17	21

8  Automatik-State (HMI_State / HMI_State_W2)	22

9  RFID Status_Code	22

10  HMI- und Testcodes	22

10.1  HMI (HM-01…07)	22

10.2  Abnahmetests T-01…T-09	22

11  Verweise	23

# 1  Zweck und Abgrenzung zum Lastenheft

Das Lastenheft beschreibt, was die Anlage leisten muss (Anforderungscodes AF, NF, HW, OB, SW, HM, SI, T). Dieses Pflichtenheft beschreibt, wie das im Repository und in TIA umgesetzt ist: SCL-Quelldateien, FUP-Netzwerke in OB1, Tags, DBs und numerische Codes (Info_Code, State, RFID Status_Code, Produktcodierung).

| Dokument | Frage | Inhalt |
| --- | --- | --- |
| Lastenheft v1.6 | Was? | Anforderungen, Scope, Abnahme T-01…T-09 |
| Pflichtenheft v1.3 | Wie? | SCL-Dateien, FUP-NW, alle PLCTags3 je Station |
| Gesamtanlage.md | Kontext | Materialfluss, Zonen, Stand |

Abweichung zum Lastenheft AF-32a (W2 Phase1 Sensor-only): Warehouse_2-Gate ist umgesetzt als Sensor UND (2a_VisionData_Combo_Done ODER 4a_RFID_Done). Automatik W2 überspringt State 10 (Reader 0) und übernimmt Produktdaten von 4a / Vision.

# 2  Traceability: Lastenheft-Code → SCL / FUP

## 2.1  Allgemein / Querschnitt

| Code | Umsetzung (SCL / TIA) | St. |
| --- | --- | --- |
| AF-01 | TIA V20 · CPU 1518F · FUP OB1 NW 1–29 · SCL-FBs | U |
| AF-02 | Factory I/O Driver-Mapping; Tags in PLC_Tags_*.csv | U |
| AF-03 | Ein TP2200 HMI_1; docs/03_Technik/HMI_Gesamtanlage.md | T |
| AF-04 | FB_Warehouse_Mode_Select / _W2 · Einricht > Auto / Hand | U |
| AF-05 | FB_Meldung / _W2 · Info_Code 0–17 · gldb_Meldungen | U |

## 2.2  Vision (AF-10…14)

| Code | Umsetzung (SCL / TIA) | St. |
| --- | --- | --- |
| AF-10 | FB_VisionReader · NW 7 · scl/Zone_2b_Vision_Foerderbaender/ | U |
| AF-11 | Gültig 1…6; 7…9 → Lid/Base_Reject_Metal | U |
| AF-12 | Color_Code 1/2/3 · Materialart=1 · ProductTyp · Artikelnummer gelatcht | U |
| AF-13 | RFID_CODE = YYMMDD×1000 + Materialart×100 + Color_Code | U |
| AF-14 | FB_RFID_ReadWrite Job 11 · Allow_Write := 4b_Done · ein Puls | U |
| AF-52 | FB_VisionReader_Metal · NW 5 · 2a_VisionData_Combo_Done %M56.0 | U |

## 2.3  RFID (AF-20…25)

| Code | Umsetzung (SCL / TIA) | St. |
| --- | --- | --- |
| AF-20 | FB_RFID_ReadWrite.scl · Command/Execute/Command ID/Status · Hold ≥100 ms | U |
| AF-21 | Index 0 RFID_CODE · 1 Artikelnummer · 2 Materialart · 3 ProductTyp | U |
| AF-22 | DB_1 Reader 2 Write (4B) · DB_2 Reader 5 Read (5B) · 4a DB_103 Reader 1 | U |
| AF-23 | Pallet_Tagged %M40.2 / %M22.5 · Combo_Done %M40.0 (Kunststoff) | U |
| AF-24 | NW 25 Write · NW 29 Read · Metall NW 16/21 OB1_RFID_Metal.scl | U |
| AF-25 | Status_Code 0/1/10 · HMI Reset %M30.5 / %M61.5 / %M71.5 | U |

## 2.4  Hochregallager (AF-30…38)

| Code | Umsetzung (SCL / TIA) | St. |
| --- | --- | --- |
| AF-30 | W1 NW 29 · gldb_LagerverwaltungData.Fach[1..54] · UDT_Fach | U |
| AF-30a | W2 NW 28 · *_W2 DBs/Tags · Stacker Crane 1 · %M70+ | U |
| AF-31 | UDT_Fach: Materialart, Color, Artikel, Typ, RFID, Pos X/Z, Belegt, Gesperrt | U |
| AF-32 | FB_Freies_Fach_Suchen · Automatik States 20–60 · FB_Einlagern | U |
| AF-32a | FB_Warehouse_Gate / _W2 · Vor_Regal UND (Combo ODER Tagged/4a_Done) | U |
| AF-33 | FB_Auslagern / _W2 · FB_Suchen (RFID→Artikel→Fach) · Sequenz Automatik begrenzt | T |
| AF-34 | FB_Loeschen · FB_Suchen · HMI_Loeschen / HMI_Suchen (+ _W2) | U |
| AF-34a | Streamlit app.py · SQLite · ohne SPS-Änderung · HM-07 / T-09 | U |
| AF-35 | FB_Berechn_Offset · HMI_Raster_Berechnen · Manual Soll / Jog | U |
| AF-36 | W1 State 10 Reader 5 · W2 State 10 Durchlauf, Daten von 4a/Vision | U |
| AF-37 | Gate sperrt Auto-Einlagern ohne Write-OK / Combo | U |
| AF-38 | Mode_Auto + Paket_Fuer_Hochregal → Start ohne HMI_Start_Einlagern | U |

## 2.5  Montage, Förderer, HMI, Sicherheit

| Code | Umsetzung (SCL / TIA) | St. |
| --- | --- | --- |
| AF-40 | PickPlace_DigitalAnalog.scl · NW 10 Metall · NW 18 Kunststoff | U |
| AF-41 | NW 11 Waage 1 · NW 19 Waage 2 | T |
| AF-42 | FB_GantryPickPlace · NW 15 · OB1_GantryPickPlace.scl | U |
| AF-43 | FB_Palletizer · NW 24 · OB1_Palletizer.scl · RFID NW 25 | U |
| AF-50 | scl/Zone_1a_Metall · Zone_1b_Kunststoff · NW 1–2, 4, 6 | P |
| AF-51 | NW 3 Metallband · NW 8 Kunststoffband · NW 26/27 Lagerbänder | T |
| NF-01 | Getrennte FBs je Zone / Querschnitt (Vision, RFID, Lager, P&P) | U |
| NF-07 | Web schreibt nicht gldb_LagerverwaltungData · HMI_Suchen bleibt SPS | U |
| HW-01 | Hardware_SPS.md · CPU 1518F · TP2200 | U |
| HW-02 / OB-01 | OB100 vorgesehen, noch nicht im Sweep — SI-05 Soll | P |
| SW-01 | FUP OB1 NW 1–29 sequenziell · SCL nur in FBs · PLC_Networks.md | U |
| HM-01…07 | HMI_Tags_*.csv · Übersicht Prod/Tag · Web Streamlit (HM-07) | T |
| SI-01 | HMI_Stop / Not_Aus · Automatik unterbricht | U |
| SI-02 | FB_Warehouse_Mode_Select: Einricht räumt Auto/Hand | U |

# 3  Sprachen: SCL und FUP

Zwei Programmiersprachen in TIA Portal: Die Ablauf- und Rechenlogik liegt in SCL (Structured Control Language, Quelle im Git). Das zyklische Hauptprogramm OB1 ist in FUP (Funktionsplan / FBD) verdrahtet: jedes Netzwerk ruft die zugehörigen SCL-FBs auf und verbindet Ein-/Ausgänge. Keine doppelte Logik in FUP — nur Aufrufe und Glue.

| Sprache | TIA-Name | Wo | Inhalt |
| --- | --- | --- | --- |
| SCL | SCL / ST | scl/**/*.scl (Git) | FBs, Sequenzen, Formeln, States |
| FUP | FBD / Funktionsplan | TIA OB1 NW 1–29 | FB-Aufrufe, I/O-Verdrahtung |
| UDT | PLC-Datentyp | scl/**/*.udt.txt | Fach, Raster, HMI, RFID-Produkt |

# 4  FUP-Codes (OB1-Netzwerke NW 1–29)

FUP-Code = Netzwerknummer in Main OB1. Titel laut TIA (PLC_Networks.md). Zone 1 CNC ohne SCL-FB (P). Legacy-Ordner scl/Zone3_PickPlace und scl/Zone4_Palettierer nicht in OB1 verdrahten.

| FUP | Zone | TIA-Netzwerktitel (kurz) | SCL-Aufruf |
| --- | --- | --- | --- |
| NW 1 | 1A | Metal Raw Material input | — (P) |
| NW 2 | 1A | Metal Robot (CNC) station | — (P) |
| NW 3 | 2A | Metal Conveyor Belts after CNC | — (T) |
| NW 4 | 1B | Plastic Raw Material input | — (P) |
| NW 5 | 2A | VisionData Sensors (Metall) | FB_VisionReader_Metal · OB1_NW5_Metal_Vision.scl |
| NW 6 | 1B | Plastic Robot (CNC) station | — (P) |
| NW 7 | 2B | VisionData Sensors (Kunststoff) | FB_VisionReader.scl (FB17) |
| NW 8 | 2B | Plastic Conveyor Belts after CNC | — (T) |
| NW 9 | 3A | Metal belts before P&P | — (T) |
| NW 10 | 3A | Metal Pick And Place 1 | PickPlace_DigitalAnalog.scl |
| NW 11 | 3A | Metal Waage 1 | — (T) |
| NW 12 | 3A | Metal belts after scale | — (T) |
| NW 13–14 | 4A | Belts / roller before palletizer | — (T) |
| NW 15 | 4A | Metal Assembly Palletizer | FB_GantryPickPlace · OB1_GantryPickPlace.scl |
| NW 16 | 4A | Metal Components RFID | OB1_RFID_Metal.scl · FB_RFID_ReadWrite |
| NW 17 | 3B | Plastic belts before P&P | — (T) |
| NW 18 | 3B | Plastic Pick and Place 2 | PickPlace_DigitalAnalog.scl (2. Instanz) |
| NW 19 | 3B | Plastic scale 2 | — (T) |
| NW 20 | 3B | Plastic belts after scale | — (T) |
| NW 21 | 4A | Metal RFID (zusätzlich) | wie NW 16 |
| NW 22–23 | 4B | Belts / roller before palletizer | — (T) |
| NW 24 | 4B | Plastic Assembly Palletizer | FB_Palletizer · OB1_Palletizer.scl |
| NW 25 | 4B | Plastic RFID Write Reader 2 | FB_RFID_ReadWrite · OB1_RFID_at_PickPlace.scl |
| NW 26 | 5A | Belts to plastic warehouse | Zone_5a_Foerderband_Lager (T) |
| NW 27 | 5B | Belts to metal warehouse | OB1_NW27_Belts_to_Metal_Warehouse.scl |
| NW 28 | 5A | Metal Components Warehouse W2 | OB1_NW28_Metal_Warehouse.scl + FBs *_W2 |
| NW 29 | 5B | Plastic Components Warehouse W1 | FUP-Aufrufe laut WAREHOUSE_1_SETUP.md §3 |

## 4.1  Stationen — Übersicht SCL / FUP / Tags

TIA-Export PLCTags3.xlsx. FIO = Prozessabbild %E/%A/%ED/%AD ↔ Factory I/O Driver. PLC = Merker %M. Alle Tags stehen in Abschnitt 4.2 (eine Tabelle Factory I/O und eine Tabelle SPS je Station) sowie in Station_PLC_FIO_Tags.md / PLCTags3_live.csv.

| Station | FUP | SCL (Dateien) | Factory I/O (Kennwerte) | SPS (Kennwerte) |
| --- | --- | --- | --- | --- |
| 1A Metall Roh | NW 1–2 | kein FB — scl/Zone_1a_Metall/ (P) | Emitter %A0.0/.1 · Sensor %E0.0/.1 · Band %A0.2/.3 | — |
| 1B Kunststoff Roh | NW 4, 6 | kein FB — scl/Zone_1b_Kunststoff/ (P) | Emitter %A1.1/.2 · Sensor %E1.0/.1 · Band %A1.0/.3 | — |
| 2A Band + Vision Metall | NW 3, 5 | FB_VisionReader_Metal.scl · OB1_NW5_Metal_Vision.scl | Bänder %A2.x/%E2.x · Vision Lid/Base %ED170/%ED174 | Combo_Done %M56.0 · RFID_CODE %MD276 |
| 2B Band + Vision Kunststoff | NW 7–8 | FB_VisionReader.scl | Bänder %A3.x/%E3.x · Vision Lid/Base %ED142/%ED146 | Combo_Done %M40.0 · Color %MW50 |
| 3A 2-axis P&P + Waage 1 | NW 9–12 | PickPlace_DigitalAnalog.scl | P&P Moving/Grab %E5/%A4–5 · Ist X/Z %ED30/%ED34 · Soll %AD16/%AD20 · Waage %ED38 | 3a_Material_Placed %M0.2 |
| 3B 2-axis P&P + Waage 2 | NW 17–20 | PickPlace_DigitalAnalog.scl (2. Instanz) | Ist X/Z %ED54/%ED58 · Soll %AD28/%AD32 · Waage 2 %ED62 | 3b_Pick_and_Place_Done %M0.3 |
| 4A 3-axis P&P + RFID Reader 1 | NW 13–16, 21 | FB_GantryPickPlace.scl · OB1_GantryPickPlace.scl · OB1_RFID_Metal.scl | P&P Ist X/Y/Z %ED42/46/50 · Soll %AD40/44/48 · Reader 1 Status %ED102 Execute %A9.0 | 4a_HMI_* %M8 · 4a_RFID_Code_Out %MD236 |
| 4B Palettierer + RFID Reader 2 | NW 22–25 | FB_Palletizer.scl · FB_RFID_ReadWrite.scl · OB1_Palletizer.scl | P&P Ist %ED86/90/94 · Soll %AD60/64/68 · Reader 2 Status %ED130 Execute %A11.1 | Pallet_Tagged / Vision %M40 |
| 5A Band + Warehouse_2 Crane 1 | NW 26, 28 | Hochregal_Automatik_Betrieb_W2.scl · OB1_NW28_Metal_Warehouse.scl · FB_*_W2.scl | Bänder %A12/%E12 · Crane 1 Ist %ED178/%ED182 Soll %AD144/%AD148 · Reader 0 %ED114 %A15.4 | Mode/HMI %M70+ · HMI_State_W2 %MW228 |
| 5B Band + Warehouse_1 Crane 0 | NW 27, 29 | Hochregal_Automatik_Betrieb.scl · FB_Warehouse_*.scl · OB1_NW27_….scl | Bänder %A13/%E13 · Crane 0 Ist %ED162/%ED166 Soll %AD136/%AD140 · Reader 5 %ED150 %A13.7 | Mode/HMI %M60+ · HMI_State %MW128 |

## 4.2  Stationen — alle Factory-I/O- und SPS-Tags

485 Tags aus PLCTags3_live.csv. Je Station: SCL-Dateien, dann vollständige Factory-I/O-Tabelle (%E/%A/%ED/%AD) und SPS-Tabelle (%M).

### Station 1A — NW 1–2 — 6 Tags (FIO 6 / SPS 0)

SCL: (kein FB — geplant) scl/Zone_1a_Metall/

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 1a_Emitter_1_Emit | %A0.0 | Bool | Standard-Variablentabelle | 1a ¦ Factory I/O: 1a_Emitter 1 (Emit) |
| 1a_Emitter_2_Emit | %A0.1 | Bool | Standard-Variablentabelle | 1a ¦ Factory I/O: 1a_Emitter 2 (Emit) |
| 1a_M_C_1_AN_WESEND_Belt_Conveyor_2m | %A0.2 | Bool | Standard-Variablentabelle | 1a ¦ Factory I/O: 1a_M_C_1_AN_WESEND_Belt Conveyor (2m) |
| 1a_M_C_ANWESEND_Belt_Conveyor_2m | %A0.3 | Bool | Standard-Variablentabelle | 1a ¦ Factory I/O: 1a_M_C_ANWESEND_Belt Conveyor (2m) |
| 1a_M_C_0_ANWESEND_Sensor | %E0.0 | Bool | Standard-Variablentabelle | 1a ¦ Factory I/O: 1a_M_C_0_ANWESEND_Sensor |
| 1a_M_C_1_ANWESEND_Sensor | %E0.1 | Bool | Standard-Variablentabelle | 1a ¦ Factory I/O: 1a_M_C_1_ANWESEND_Sensor |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| — | — | — | keine SPS-Merker |  |

### Station 1B — NW 4, 6 — 6 Tags (FIO 6 / SPS 0)

SCL: (kein FB — geplant) scl/Zone_1b_Kunststoff/

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 1b_DC_0_Belt_Conveyor_2m | %A1.0 | Bool | Standard-Variablentabelle | 1b ¦ Factory I/O: 1b_DC_0_Belt_Conveyor(2m) |
| 1b_Emitter_5_Emit | %A1.1 | Bool | Standard-Variablentabelle | 1b ¦ Factory I/O: 1b_Emitter 5 (Emit) |
| 1b_raw_material_emitter_2 | %A1.2 | Bool | Standard-Variablentabelle | 1b ¦ Factory I/O: 1b_raw_material_emitter__2 |
| 1b_DC_1_Belt Conveyor (2m) | %A1.3 | Bool | Forderband |  |
| 1b_plastic_raw_material_sensor_1 | %E1.0 | Bool | Standard-Variablentabelle | 1b ¦ Factory I/O: 1b_plastic_raw_material_sensor_1 |
| 1b_Plastic_raw_material_sensor_2 | %E1.1 | Bool | Standard-Variablentabelle | 1b ¦ Factory I/O: 1b_Plastic_raw_material_sensor_2 |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| — | — | — | keine SPS-Merker |  |

### Station 2A — NW 3, 5 — 31 Tags (FIO 12 / SPS 19)

SCL: scl/Zone_2a_Foerderbaender/FB_VisionReader_Metal.scl · scl/Zone_2a_Foerderbaender/OB1_NW5_Metal_Vision.scl

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 2a_After_DC_Our_0 | %A2.0 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_After_DC_Our_0 |
| 2a_After_MC_out_0 | %A2.1 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_After_MC_out_0 |
| 2a_After_Metal_MC_Base_Conveyor | %A2.2 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_After_Metal_MC_Base_Conveyor |
| 2a_After_Metal_MC_Lid_Conveyor | %A2.3 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_After_Metal_MC_Lid_Conveyor |
| 2a_Motor_Before_PP_Base | %A2.4 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_Motor_Before_PP_Base |
| 2a_Motor_Before_PP_Lid | %A2.5 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_Motor_Before_PP_Lid |
| 2a_Eck_Foerderband_MBase | %E2.0 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_Eck_Foerderband_MBase |
| 2a_Eck_Foerderband_MLid | %E2.1 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_Eck_Foerderband_MLid |
| 2a_Worked_Part_from_MC_out | %E2.2 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_Worked_Part_from_MC_out |
| 2a_Worked_Part_from_MC_out_2 | %E2.3 | Bool | Standard-Variablentabelle | 2a ¦ Factory I/O: 2a_Worked_Part_from_MC_out_2 |
| 2a_Vision Sensor Lid(Value) | %ED170 | DInt | 2a_VisionDataSensors |  |
| 2a_Vision Sensor Base(Value) | %ED174 | DInt | 2a_VisionDataSensors |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 2a_VisionData_Write_Req | %M20.1 | Bool | 2b_VisionDataSensors |  |
| 2a_Metal_VisionData_Combo_Done | %M56.0 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_Both_Ready | %M56.1 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_Busy | %M56.2 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_Error | %M56.3 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_Vision_Lid_Valid | %M56.4 | Bool | 2a_VisionDataSensors | Code=8 |
| 2a_Metal_Vision_Base_Valid | %M56.5 | Bool | 2a_VisionDataSensors | Code=9 |
| 2a_Metal_Vision_Lid_Reject_Plastic | %M56.6 | Bool | 2a_VisionDataSensors | Code 1..6 |
| 2a_Metal_Vision_Base_Reject_Plastic | %M56.7 | Bool | 2a_VisionDataSensors | Code 1..6 |
| 2a_Metal_HMI_RFID_Write | %M57.0 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_Vision_Reset | %M57.1 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_Waiting_For_Write | %M57.2 | Bool | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_Materialart | %MB284 | USInt | 2a_VisionDataSensors | Always 2=Metall |
| 2a_Metal_VisionData_Prod_Month | %MB288 | USInt | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_Prod_Day | %MB289 | USInt | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_RFID_CODE | %MD276 | UDInt | 2a_VisionDataSensors | Production date YYMMDD |
| 2a_Metal_VisionData_Artikelnummer | %MW250 | UInt | 2a_VisionDataSensors | Sequential article |
| 2a_Metal_VisionData_Prod_Year | %MW286 | UInt | 2a_VisionDataSensors |  |
| 2a_Metal_VisionData_State | %MW294 | Int | 2a_VisionDataSensors | 1 sense 2 write 3 done 4 wait |

### Station 2B — NW 7–8 — 31 Tags (FIO 13 / SPS 18)

SCL: scl/Zone_2b_Vision_Foerderbaender/FB_VisionReader.scl

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 2b_After_Plastic_DC_out_0 | %A3.0 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_After_Plastic_DC_out_0 |
| 2b_After_Plastic_MC_out_1 | %A3.1 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_After_Plastic_MC_out_1 |
| 2b_After_Plastic_MC_out_2 | %A3.2 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_After_Plastic_MC_out_2 |
| 2b_Curved_Belt_Conveyor_6_CW | %A3.3 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_Curved Belt Conveyor 6 CW ¦ DUPLICATE source name - verify mapping |
| 2b_Curved_Belt_Conveyor_6_CW_2 | %A3.4 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_Curved Belt Conveyor 6 CW ¦ DUPLICATE source name - verify mapping |
| 2b_Curved_Belt_Conveyor_8_C2 | %A3.5 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_Curved Belt Conveyor 8 C2 |
| 2b_Eck_Foerderband_MLid | %A3.6 | Bool | Forderband |  |
| 2b_Worked_Part_from_MC_out_1 | %E3.0 | Bool | Standard-Variablentabelle |  |
| 2b_Diffuse_Sensor_39 | %E3.1 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_Diffuse Sensor 39 |
| 2b_Diffuse_Sensor_42 | %E3.2 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_Diffuse Sensor 42 |
| 2b_Worked_Part_from_MC_out_2 | %E3.3 | Bool | Standard-Variablentabelle |  |
| 2b_Vision Sensor Lid (Value) | %ED142 | DInt | 2b_VisionDataSensors |  |
| 2b_Vision Sensor Base(Value) | %ED146 | DInt | 2b_VisionDataSensors |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 2b_HMI_RFID_Write | %M20.0 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Bereit | %M30.0 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Busy | %M30.1 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Error | %M30.2 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Gueltig | %M30.3 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Done | %M30.4 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Reset | %M30.5 | Bool | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Fehler | %M30.6 | Bool | 2b_VisionDataSensors |  |
| 2b_VisionData_Combo_Done | %M40.0 | Bool | 2b_VisionDataSensors |  |
| 2b_VisionData_Both_Ready | %M40.1 | Bool | Forderband |  |
| 2b_HMI_RFID_Pallet_Tagged | %M40.2 | Bool | Standard-Variablentabelle |  |
| 2b_VisionData_Materialart | %MB46 | USInt | 2b_VisionDataSensors |  |
| 2b_VisionData_RFID_CODE | %MD42 | UDInt | 2b_VisionDataSensors |  |
| 2b_VisionData_Artikelnumber | %MW28 | UInt | 2b_VisionDataSensors |  |
| 2b_VisionData_ProductType | %MW38 | UInt | 2b_VisionDataSensors |  |
| 2b_HMI_RFID_Status_Code | %MW44 | Int | 2b_VisionDataSensors |  |
| 2b_VisionData_State | %MW48 | Int | Forderband |  |
| 2b_VisionData_Color_Code | %MW50 | Int | Forderband |  |

### Station 2A/2B Band — NW 3 / 8 / 26 / 27 — 9 Tags (FIO 4 / SPS 5)

SCL: (FUP-Bänder; SCL in der jeweiligen Zone)

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| Motor | %A0.4 | Bool | Forderband |  |
| Weight_ok | %A5.5 | Bool | Forderband |  |
| Aut_Start_Mode | %E5.5 | Bool | Forderband |  |
| Waage0_Reset | %E5.7 | Bool | Forderband |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| Material_Art_In | %MB1 | USInt | Forderband |  |
| Memory_Index | %MD2 | DInt | Forderband |  |
| RFID_Code_IN | %MD22 | UDInt | Forderband |  |
| ProductType_In | %MW26 | UInt | Forderband |  |
| Artikelnummer_In | %MW6 | UInt | Forderband |  |

### Station 3A — NW 9–12 — 33 Tags (FIO 32 / SPS 1)

SCL: scl/Zone_3a_Metall_PickPlace/PickPlace_DigitalAnalog.scl

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 3a_Left_Positioner_2_Raise | %A4.0 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Left Positioner 2 (Raise) |
| 3a_Left_Positioner_Metal_Baugruppe_Clamp | %A4.1 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Left Positioner _Metal_Baugruppe(Clamp) |
| 3a_Motor_at_PP_Base | %A4.2 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Motor_at_PP_Base |
| 3a_Motor_at_PP_Lid | %A4.3 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Motor_at_PP_Lid |
| 3a_Nach_Waage_Curved_Belt_Conveyor_0_CW | %A4.4 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Nach_Waage_Curved Belt Conveyor 0 CW |
| 3a_Right_Lid_Positioner_0_Clamp | %A4.5 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Right_Lid_Positioner 0 (Clamp) |
| 3a_Right_Lid_Positioner_0_Raise | %A4.6 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Right_Lid_Positioner 0 (Raise) |
| 3a_Two_Axis_Pick_and_Place_0_Grab | %A4.7 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 (Grab) |
| 3a_Two_Axis_Pick_and_Place_0_Rotate_CW | %A5.0 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 Rotate CW |
| 3a_Two_Axis_Pick_and_Place_0_Gripper_CW | %A5.1 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 Gripper CW |
| 3a_Two_Axis_Pick_and_Place_0_Rotate_CCW | %A5.2 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 Rotate CCW |
| 3a_Two_Axis_Pick_and_Place_0_Gripper_CCW | %A5.3 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 Gripper CCW |
| 3a_Waage_Conveyor_Scale_0_Plus | %A5.4 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Waage_Conveyor Scale 0 (+) |
| 3a_Curved Belt Conveyor 5 CW | %A5.7 | Bool | Forderband |  |
| 3a_Two_Axis_Pick_and_Place_0_X_Set_Point_V | %AD16 | Real | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 X Set Point (V) |
| 3a_Two_Axis_Pick_and_Place_0_Z_Set_Point_V | %AD20 | Real | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 Z Set Point (V) |
| 3a_Baugruppe_Left_Positioner_2_Clamped | %E4.0 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Baugruppe_Left Positioner 2 (Clamped) |
| 3a_Part_Left_1 | %E4.1 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Part_Left_1 |
| 3a_Right_Positioner_0_Limit | %E4.2 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Right Positioner 0 (Limit) |
| 3a_Right_Positioner_0_Clamped | %E4.3 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Right Positioner 0 (Clamped) |
| 3a_Sensor_at_PP_Base | %E4.4 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Sensor_at_PP_Base |
| 3a_Sensor_at_PP_Lid | %E4.5 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Sensor_at_PP_Lid |
| 3a_Sensor_Before_PP_Base | %E4.6 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Sensor_Before_PP_Base |
| 3a_Sensor_Before_PP_Lid | %E4.7 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Sensor_Before_PP_Lid |
| 3a_Two_Axis_Pick_and_Place_0_Moving_X | %E5.0 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 (Moving X) |
| 3a_Two_Axis_Pick_and_Place_0_Moving_Z | %E5.1 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 (Moving Z) |
| 3a_Two_Axis_Pick_and_Place_0_Rotating | %E5.2 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 (Rotating) |
| 3a_Two_Axis_Pick_and_Place_0_Item_Detected | %E5.3 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 (Item Detected) |
| 3a_Two_Axis_Pick_and_Place_0_Gripper_Rotating | %E5.4 | Bool | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 (Gripper Rotating) |
| 3a_Two_Axis_Pick_and_Place_0_X_Position_V | %ED30 | Real | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 X Position (V) |
| 3a_Two_Axis_Pick_and_Place_0_Z_Position_V | %ED34 | Real | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Two-Axis Pick & Place 0 Z Position (V) |
| 3a_Waage_0_Weight_V | %ED38 | Real | Standard-Variablentabelle | 3a ¦ Factory I/O: 3a_Waage 0 Weight (V) |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 3a_Material_Placed_PP_Metal | %M0.2 | Bool | Standard-Variablentabelle |  |

### Station 3B — NW 17–20 — 36 Tags (FIO 35 / SPS 1)

SCL: scl/Zone_3a_Metall_PickPlace/PickPlace_DigitalAnalog.scl (2. Instanz, NW 18)

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 3b_Belt_Conveyor_2m_6 | %A6.0 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Belt Conveyor (2m) 6 |
| 3b_Belt_Conveyor_2m_13 | %A6.1 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Belt Conveyor (2m) 13 |
| 3b_Conveyor_Scale_2_Plus | %A6.2 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Conveyor Scale 2 (+) |
| 3b_Conveyor_Scale_2 | %A6.3 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Conveyor Scale 2 (-) |
| 3b_Left_Positioner_1_Clamp | %A6.4 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Left Positioner 1 (Clamp) |
| 3b_Left_Positioner_1_Raise | %A6.5 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Left Positioner 1 (Raise) ¦ DUPLICATE source name - verify mapping |
| 3b_Right_Positioner_3_Clamp | %A6.6 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Right Positioner 3 (Clamp) |
| 3b_Right_Positioner_1_Raise | %A6.7 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Left Positioner 1 (Raise) ¦ DUPLICATE source name - verify mapping |
| 3b_Two_Axis_Pick_and_Place_1_Grab | %A7.0 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 (Grab) |
| 3b_Two_Axis_Pick_and_Place_1_Rotate_CW | %A7.1 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 Rotate CW |
| 3b_Two_Axis_Pick_and_Place_1_Rotate_CCW | %A7.2 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 Rotate CCW ¦ DUPLICATE source name - verify mapping |
| 3b_Two_Axis_Pick_and_Place_1_Rotate_CCW_2 | %A7.3 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 Rotate CCW ¦ DUPLICATE source name - verify mapping |
| 3b_Two_Axis_Pick_and_Place_1_Gripper_CCW | %A7.4 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 Gripper CCW |
| 3b_stack_light | %A7.5 | Bool | Forderband |  |
| 3b_NAch_Waage_Curved_Belt | %A7.6 | Bool | Forderband |  |
| 3b_Two-Axis Pick & Place 1 X Set Point (V) | %AD28 | Real | Forderband |  |
| 3b_Two-Axis Pick & Place 1 Z Set Point (V) | %AD32 | Real | Forderband |  |
| 3b_Diffuse_Sensor_36 | %E6.0 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Diffuse Sensor 36 |
| 3b_Diffuse_Sensor_38 | %E6.1 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Diffuse Sensor 38 |
| 3b_Diffuse_Sensor_40 | %E6.2 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Diffuse Sensor 40 |
| 3b_Left_Positioner_1_Limit | %E6.3 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Left Positioner 1 (Limit) |
| 3b_Left_Positioner_1_Clamped | %E6.4 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Left Positioner 1 (Clamped) |
| 3b_Right_Positioner_3_Limit | %E6.5 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Right Positioner 3 (Limit) |
| 3b_Right_Positioner_3_Clamped | %E6.6 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Right Positioner 3 (Clamped) |
| 3b_Two_Axis_Pick_and_Place_1_Moving_X | %E6.7 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 (Moving X) |
| 3b_Two_Axis_Pick_and_Place_1_Moving_Z | %E7.0 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 (Moving Z) |
| 3b_Two_Axis_Pick_and_Place_1_Rotating | %E7.1 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 (Rotating) |
| 3b_Two_Axis_Pick_and_Place_1_Item_Detected | %E7.2 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 (Item Detected) |
| 3b_Two_Axis_Pick_and_Place_1_Gripper_Rotating | %E7.3 | Bool | Standard-Variablentabelle | 3b ¦ Factory I/O: 3b_Two-Axis Pick & Place 1 (Gripper Rotating) |
| 3b_Diffuse_Sensor_12 | %E7.4 | Bool | Standard-Variablentabelle | 2b ¦ Factory I/O: 2b_Diffuse Sensor 12 |
| 3b_Diffuse_Sensor_0 | %E7.5 | Bool | Standard-Variablentabelle |  |
| 3b_Diffuse_Sensor_11 | %E7.6 | Bool | Standard-Variablentabelle |  |
| 3b_Two-Axis Pick & Place 1 X Position (V) | %ED54 | Real | Forderband |  |
| 3b_Two-Axis Pick & Place 1 Z Position (V) | %ED58 | Real | Forderband |  |
| 3b_Waage 2 Weight (V) | %ED62 | Real | Forderband |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 3b_Pick_and_Place_Done | %M0.3 | Bool | Forderband |  |

### Station 4A — NW 13–16, 21 — 83 Tags (FIO 30 / SPS 53)

SCL: scl/Zone_4a_Metall_Palletizer_RFID/FB_GantryPickPlace.scl · scl/Zone_4a_Metall_Palletizer_RFID/OB1_GantryPickPlace.scl · scl/Zone_4a_Metall_Palletizer_RFID/OB1_RFID_Metal.scl · scl/Zone_4b_Kunststoff_Palletizer_RFID/FB_RFID_ReadWrite.scl (Instanz Metall)

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 4a_Warning_Light_1 | %A8.0 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Warning Light 1 |
| 4a_Pick_and_Place_1_C_Plus | %A8.1 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 C(+) |
| 4a_Pick_and_Place_1_Grab | %A8.2 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 (Grab) |
| 4a_Belt_Conveyor_at_palletizer_4m_7 | %A8.3 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Belt _Conveyor_at_palletizer (4m) 7 |
| 4a_Palletizer_Belt_Conveyor_4m_0 | %A8.4 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Palletizer_Belt Conveyor (4m) 0 |
| 4a_Roller_Conveyor_6m_4 | %A8.5 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Roller Conveyor (6m) 4 |
| 4a_Roller Conveyor (2m) 2 | %A8.6 | Bool | Forderband |  |
| 4a_Emitt_Pallet | %A8.7 | Bool | Forderband |  |
| 4a_RFID Reader 1 Execute Command | %A9.0 | Bool | 4a_4b_RFID_Readers |  |
| 4a_Pick_and_Place_1_X_Set_Point_V | %AD40 | Real | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 X Set Point (V) |
| 4a_Pick_and_Place_1_Y_Set_Point_V | %AD44 | Real | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 Y Set Point(V) |
| 4a_Pick_and_Place_1_Z_Set_Point_V | %AD48 | Real | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 Z Set Point (V) |
| 4a_RFID Reader 1 Command | %AD76 | DInt | 4a_4b_RFID_Readers |  |
| 4a_RFID Reader 1 Write Data | %AD80 | DInt | 4a_4b_RFID_Readers |  |
| 4a_RFID Reader 1 Memory Index | %AD84 | DInt | 4a_4b_RFID_Readers |  |
| 4a_at_Palletizer_machine | %E8.0 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_at_palletierer machine ¦ DUPLICATE source name - verify mapping |
| 4a_vor_Palletizer_Band_set | %E8.1 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_vor_Palletizer_Band_set |
| 4a_Pallet_at_Palletizer | %E8.2 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pallet_at_Palletizer |
| 4a_at_Palletizer_machine_2 | %E8.3 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_at_palletierer machine ¦ DUPLICATE source name - verify mapping |
| 4a_Pallet_Present | %E8.4 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pallet_Present |
| 4a_Pick_and_Place_1_Moving_Z | %E8.5 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 (Moving-Z) |
| 4a_Pick_and_Place_1_Moving_XY | %E8.6 | Bool | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 (Moving-XY) |
| 4a_pick_and_place__C_Limit | %E8.7 | Bool | Forderband |  |
| 4a_Pick & Place 1 (Box Detected) | %E9.0 | Bool | Standard-Variablentabelle |  |
| 4a_RFID Reader 1 Status | %ED102 | DInt | 4a_4b_RFID_Readers |  |
| 4a_RFID Reader 1 Read Data | %ED106 | DInt | 4a_4b_RFID_Readers |  |
| 4a_RFID Reader 1 Command ID | %ED110 | DInt | 4a_4b_RFID_Readers |  |
| 4a_Pick_and_Place_1_X_Position_V | %ED42 | Real | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 X Position (V) |
| 4a_Pick_and_Place_1_Y_Position_V | %ED46 | Real | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 Y Position (V) |
| 4a_Pick_and_Place_1_Z_Position_V | %ED50 | Real | Standard-Variablentabelle | 4a ¦ Factory I/O: 4a_Pick & Place 1 Z Position (V) |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 4b_Done(1) | %M10.0 | Bool | Gantry_2 |  |
| 4b_HMI_Lamp_Ready(1) | %M10.1 | Bool | Gantry_2 |  |
| 4b_HMI_Lamp_Running(1) | %M10.2 | Bool | Gantry_2 |  |
| 4a_Pallet_Ready | %M10.3 | Bool | Gantry | HMI compatibility (palletizer name) |
| 4a_Loading_Pallet_Active | %M10.4 | Bool | Gantry | TRUE while Grab |
| 4a_Busy | %M10.5 | Bool | Gantry | Zone 4A Busy |
| 4a_Done | %M10.6 | Bool | Gantry | Zone 4A Done |
| 4a_HMI_Lamp_Ready | %M11.0 | Bool | Gantry | Zone 4A |
| 4a_HMI_Lamp_Running | %M11.1 | Bool | Gantry | Zone 4A |
| 4a_HMI_Lamp_Done | %M11.2 | Bool | Gantry | Zone 4A |
| 4a_HMI_Lamp_Stopped | %M11.3 | Bool | Gantry | Zone 4A |
| 4a_HMI_Lamp_Auto | %M11.4 | Bool | Gantry | Zone 4A |
| 4a_HMI_Lamp_Manual | %M11.5 | Bool | Gantry | Zone 4A |
| 4b_HMI_Lamp_Done(1) | %M12.3 | Bool | Gantry_2 |  |
| 4b_HMI_Lamp_Stopped(1) | %M12.4 | Bool | Gantry_2 |  |
| 4b_HMI_Lamp_Auto(1) | %M12.5 | Bool | Gantry_2 |  |
| 4b_HMI_Lamp_Manual(1) | %M12.6 | Bool | Gantry_2 |  |
| 4a_RFID_Busy | %M21.0 | Bool | 4a_4b_RFID_Readers |  |
| 4a_RFID_Done | %M21.1 | Bool | 4a_4b_RFID_Readers |  |
| 4a_RFID_Error | %M21.2 | Bool | 4a_4b_RFID_Readers |  |
| 4a_RFID_Tag_Present | %M21.3 | Bool | 4a_4b_RFID_Readers |  |
| 4a_RFID_Metal_Bereit | %M34.0 | Bool | 4a_4b_RFID_Readers |  |
| 4a_RFID_Metal_Gueltig | %M34.1 | Bool | 4a_4b_RFID_Readers |  |
| 4a_RFID_Metal_Fehler | %M34.2 | Bool | 4a_4b_RFID_Readers |  |
| 4a_HMI_Start | %M8.0 | Bool | Gantry | Zone 4A HMI (same as previous Palletizer screen) |
| 4a_HMI_Stop | %M8.1 | Bool | Gantry | Zone 4A |
| 4a_HMI_Reset | %M8.2 | Bool | Gantry | Zone 4A |
| 4a_HMI_Auto | %M8.3 | Bool | Gantry | Zone 4A |
| 4a_HMI_SingleCycle | %M8.4 | Bool | Gantry | Zone 4A |
| 4b_HMI_Start(1) | %M9.0 | Bool | Gantry_2 |  |
| 4b_HMI_Stop(1) | %M9.1 | Bool | Gantry_2 |  |
| 4b_HMI_Reset(1) | %M9.2 | Bool | Gantry_2 |  |
| 4b_HMI_Auto(1) | %M9.3 | Bool | Gantry_2 |  |
| 4b_HMI_SingleCycle(1) | %M9.4 | Bool | Gantry_2 |  |
| 4b_Pallet_Ready(1) | %M9.5 | Bool | Gantry_2 |  |
| 4b_Loading_Pallet_Active(1) | %M9.6 | Bool | Gantry_2 |  |
| 4b_Busy(1) | %M9.7 | Bool | Gantry_2 |  |
| 4a_RFID_Code_Out | %MD236 | UDInt | 4a_4b_RFID_Readers |  |
| 4a_RFID_Data_Out | %MD240 | UDInt | 4a_4b_RFID_Readers |  |
| 4a_RFID_Serial_Number | %MD244 | UDInt | 4a_4b_RFID_Readers |  |
| 4a_State | %MW12 | Int | Gantry | Zone 4A step |
| 4a_Assembly_Count | %MW14 | Int | Gantry | HMI compatibility |
| 4a_RFID_Artikelnummer_Out | %MW150 | UInt | 4a_4b_RFID_Readers |  |
| 4a_RFID_State | %MW152 | Int | 4a_4b_RFID_Readers |  |
| 4a_RFID_Status_Code | %MW154 | Int | 4a_4b_RFID_Readers |  |
| 4a_RFID_Metal_ProductTyp_Out | %MW156 | UInt | 4a_4b_RFID_Readers |  |
| 4a_HMI_Cycle_Count | %MW16 | Int | Gantry | Zone 4A |
| 4a_HMI_Step_Text_ID | %MW18 | Int | Gantry | Zone 4A |
| 4a_RFID_RFID_Materialart_Out | %MW182 | UInt | 4a_4b_RFID_Readers |  |
| 4b_State(1) | %MW30 | Int | Gantry_2 |  |
| 4b_HMI_Cycle_Count(1) | %MW32 | Int | Gantry_2 |  |
| 4b_Assembly_Count(1) | %MW34 | Int | Gantry_2 |  |
| 4b_HMI_Step_Text_ID(1) | %MW36 | Int | Gantry_2 |  |

### Station 4B — NW 22–25 — 29 Tags (FIO 29 / SPS 0)

SCL: scl/Zone_4b_Kunststoff_Palletizer_RFID/FB_Palletizer.scl · scl/Zone_4b_Kunststoff_Palletizer_RFID/OB1_Palletizer.scl · scl/Zone_4b_Kunststoff_Palletizer_RFID/UDT_Palletizer.scl · scl/Zone_4b_Kunststoff_Palletizer_RFID/FB_RFID_ReadWrite.scl · scl/Zone_4b_Kunststoff_Palletizer_RFID/OB1_RFID_at_PickPlace.scl · scl/Zone_4b_Kunststoff_Palletizer_RFID/OB1_Vision_RFID_before_Pallet.scl

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 4b_Belt Conveyor (4m) 6 | %A10.0 | Bool | Forderband |  |
| 4b_Pick & Place 1 C(+) | %A10.2 | Bool | Standard-Variablentabelle |  |
| 4b_Pick & Place 1 (Grab) | %A10.3 | Bool | Standard-Variablentabelle |  |
| 4b_Roller Conveyor (2m) 3 | %A10.4 | Bool | Forderband |  |
| 4b_Roller Conveyor (6m) 1 | %A10.5 | Bool | Forderband |  |
| 4b_Stacker_BeltConveyor_Plastic(6m) | %A10.6 | Bool | Standard-Variablentabelle |  |
| 4b_Warning_Light_6 | %A10.7 | Bool | Standard-Variablentabelle |  |
| 4b_RFID Reader 2 Execute Command | %A11.1 | Bool | Forderband |  |
| 4b_RFID Reader 2 Command | %AD104 | DInt | 4a_4b_RFID_Readers |  |
| 4b_RFID Reader 2 Write Data | %AD108 | DInt | 4a_4b_RFID_Readers |  |
| 4b_RFID Reader 2 Memory Index | %AD112 | DInt | 4a_4b_RFID_Readers |  |
| 4b_Pick & Place 1 Y Set Point(V) | %AD60 | Real | Standard-Variablentabelle |  |
| 4b_Pick & Place 1 X Set Point (V) | %AD64 | Real | Standard-Variablentabelle |  |
| 4b_Pick & Place 1 Z Set Point (V) | %AD68 | Real | Standard-Variablentabelle |  |
| 4b_Diffuse_Sensor_10 | %E10.0 | Bool | Standard-Variablentabelle |  |
| 4b_Diffuse_Sensor_26 | %E10.1 | Bool | Forderband |  |
| 4b_Diffuse Sensor 41 | %E10.2 | Bool | Standard-Variablentabelle |  |
| 4b_Pallet_Present | %E10.3 | Bool | Forderband |  |
| 4b_Pick & Place 0 (C Limit) | %E10.4 | Bool | Standard-Variablentabelle |  |
| 4b_Pick & Place 0 (Moving-Z) | %E10.5 | Bool | Standard-Variablentabelle |  |
| 4b_Pick & Place 0 (Moving-XY) | %E10.6 | Bool | Standard-Variablentabelle |  |
| 4b_Pick & Place 0 (Box Detected) | %E10.7 | Bool | Standard-Variablentabelle |  |
| 4b_Diffuse Sensor 13 | %E11.0 | Bool | Forderband |  |
| 4b_RFID Reader 2 Status | %ED130 | DInt | 4a_4b_RFID_Readers |  |
| 4b_RFID Reader 2 Read Data | %ED134 | DInt | 4a_4b_RFID_Readers |  |
| 4b_RFID Reader 2 Command ID | %ED138 | DInt | 4a_4b_RFID_Readers |  |
| 4b_Pick & Place 0 X Position (V) | %ED86 | Real | Standard-Variablentabelle |  |
| 4b_Pick & Place 0 Y Position (V) | %ED90 | Real | Standard-Variablentabelle |  |
| 4b_Pick & Place 0 Z Position (V) | %ED94 | Real | Standard-Variablentabelle |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| — | — | — | keine SPS-Merker |  |

### Station 5A — NW 26, 28 Warehouse_2 — 108 Tags (FIO 35 / SPS 73)

SCL: scl/Zone_5a_Metall_Hochregallager/Hochregal_Automatik_Betrieb_W2.scl · scl/Zone_5a_Metall_Hochregallager/OB1_NW28_Metal_Warehouse.scl · scl/Zone_5a_Metall_Hochregallager/OB1_RFID_5a_DB4.scl · scl/Zone_5a_Metall_Hochregallager/FB_Warehouse_Mode_Select_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Warehouse_Gate_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Warehouse_Stacker_IO_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Warehouse_Manual_Soll_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Warehouse_Actuators_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Einlagern_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Auslagern_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Suchen_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Loeschen_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Freies_Fach_Suchen_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Datenverwaltung_Lager_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Berechn_Offset_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Meldung_W2.scl · scl/Zone_5a_Metall_Hochregallager/FB_Lagerstatus_W2.scl · (NW 26 Bänder → W1: kein eigenes SCL, nur FUP)

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 5a_Roller Conveyor (6m) 2 | %A12.0 | Bool | Forderband |  |
| 5a_Curved Roller Conveyor 1 CCW | %A12.1 | Bool | Forderband |  |
| 5a_Curved Roller Conveyor 7 CW | %A12.2 | Bool | Standard-Variablentabelle |  |
| 5a_Curved Roller Conveyor 0 CW | %A12.3 | Bool | Forderband |  |
| 5a_Roller Conveyor (2m) 0 | %A12.4 | Bool | Forderband |  |
| 5a_Roller Conveyor (2m) 5 | %A12.5 | Bool | Standard-Variablentabelle |  |
| 5a_Roller Conveyor (4m) 5 | %A12.6 | Bool | Forderband |  |
| 5a_Roller Conveyor (6m) 3 | %A12.7 | Bool | Forderband |  |
| 5a_Loading Conveyor | %A14.5 | Bool | Standard-Variablentabelle |  |
| 5a_Stacker Crane 1 (Left) | %A15.0 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 (Right) | %A15.1 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Warning Light 4 | %A15.2 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Alarm Siren 0 | %A15.3 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_RFID Reader 0 Execute Command | %A15.4 | Bool | 5a_RFID |  |
| 5a_Stacker Crane 1 X Set Point (V) | %AD144 | Real | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Z Set Point (V) | %AD148 | Real | HMI_Tags_Warehouse_2 |  |
| 5a_RFID Reader 0 Command | %AD88 | DInt | 5a_RFID |  |
| 5a_RFID Reader 0 Write Data | %AD92 | DInt | 5a_RFID |  |
| 5a_RFID Reader 0 Memory Index | %AD96 | DInt | 5a_RFID |  |
| 5a_to_warehouse | %E12.0 | Bool | Forderband |  |
| 5a_Diffuse Sensor 21 | %E12.1 | Bool | Forderband |  |
| 5a_Diffuse Sensor 25 | %E12.2 | Bool | Forderband |  |
| 5a_Diffuse Sensor 55 | %E12.3 | Bool | Standard-Variablentabelle |  |
| 5a_Metal_Palette_vor_regal | %E15.0 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Moving-X | %E15.1 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Moving-Z | %E15.2 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Left Limit | %E15.3 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Right Limit | %E15.4 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Middle Limit | %E15.5 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_Metal_Pallette_A_Regal | %E15.6 | Bool | Standard-Variablentabelle |  |
| 5a_RFID Reader 0 Status | %ED114 | DInt | 5a_RFID |  |
| 5a_RFID Reader 0 Read Data | %ED118 | DInt | 5a_RFID |  |
| 5a_RFID Reader 0 Command ID | %ED122 | DInt | 5a_RFID |  |
| 5a_Stacker Crane 1 X Position (V) | %ED178 | Real | HMI_Tags_Warehouse_2 |  |
| 5a_Stacker Crane 1 Z Position (V) | %ED182 | Real | HMI_Tags_Warehouse_2 |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| Mode_Einricht_W2 | %M70.0 | Bool | HMI_Tags_Warehouse_2 |  |
| Mode_Auto_W2 | %M70.1 | Bool | HMI_Tags_Warehouse_2 |  |
| Mode_Hand_W2 | %M70.2 | Bool | HMI_Tags_Warehouse_2 |  |
| Not_Aus_W2 | %M70.3 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Initialisieren_W2 | %M70.4 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Reset_Meldung_W2 | %M70.5 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Pos_Speichern_W2 | %M70.6 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Raster_Berechnen_W2 | %M70.7 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Loeschen_W2 | %M71.0 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Suchen_W2 | %M71.1 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_FreiesFach_W2 | %M71.2 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Start_Einlagern_W2 | %M71.3 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Start_Auslagern_W2 | %M71.4 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Reset_W2 | %M71.5 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Stop_W2 | %M71.6 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Copy_Ist_to_Soll_W2 | %M71.7 | Bool | HMI_Tags_Warehouse_2 |  |
| Paket_Vor_Regal_W2 | %M72.0 | Bool | HMI_Tags_Warehouse_2 |  |
| Paket_Fuer_Hochregal_W2 | %M72.1 | Bool | HMI_Tags_Warehouse_2 |  |
| Palette_Aufgenommen_W2 | %M72.2 | Bool | HMI_Tags_Warehouse_2 |  |
| Palette_Abgelegt_W2 | %M72.3 | Bool | HMI_Tags_Warehouse_2 |  |
| Position_Erricht_W2 | %M72.4 | Bool | HMI_Tags_Warehouse_2 |  |
| Gabel_Rechts_Ausgefahren_W2 | %M72.5 | Bool | HMI_Tags_Warehouse_2 |  |
| Gabel_Links_Ausgefahren_W2 | %M72.6 | Bool | HMI_Tags_Warehouse_2 |  |
| Gabel_Eingefahren_W2 | %M72.7 | Bool | HMI_Tags_Warehouse_2 |  |
| Fahre_Zu_Position_W2 | %M73.0 | Bool | HMI_Tags_Warehouse_2 |  |
| Palette_Aufnehmen_Cmd_W2 | %M73.1 | Bool | HMI_Tags_Warehouse_2 |  |
| Palette_Ablegen_Cmd_W2 | %M73.2 | Bool | HMI_Tags_Warehouse_2 |  |
| Gabel_Rechts_Cmd_W2 | %M73.3 | Bool | HMI_Tags_Warehouse_2 |  |
| Gabel_Links_Cmd_W2 | %M73.4 | Bool | HMI_Tags_Warehouse_2 |  |
| Gabel_Mitte_Cmd_W2 | %M73.5 | Bool | HMI_Tags_Warehouse_2 |  |
| HRL2_Busy | %M74.0 | Bool | HMI_Tags_Warehouse_2 |  |
| HRL2_Done | %M74.1 | Bool | HMI_Tags_Warehouse_2 |  |
| HRL2_Error | %M74.2 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Soll_X_Plus_W2 | %M75.7 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Soll_X_Minus_W2 | %M76.0 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Soll_Z_Plus_W2 | %M76.1 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Soll_Z_Minus_W2 | %M76.2 | Bool | HMI_Tags_Warehouse_2 |  |
| HMI_Copy_Ist_to_Band_W2 | %M76.3 | Bool | HMI_Tags_Warehouse_2 |  |
| 5a_RFID_Lesen | %M77.0 | Bool | 5a_RFID |  |
| 5a_RFID_Busy | %M77.1 | Bool | 5a_RFID |  |
| 5a_RFID_Done | %M77.2 | Bool | 5a_RFID |  |
| 5a_RFID_Error | %M77.3 | Bool | 5a_RFID |  |
| 5a_RFID_Bereit | %M77.4 | Bool | 5a_RFID |  |
| 5a_RFID_Gueltig | %M77.5 | Bool | 5a_RFID |  |
| 5a_RFID_Fehler | %M77.6 | Bool | 5a_RFID |  |
| 5a_RFID_Materialart_Out | %MB286 | USInt | 5a_RFID |  |
| HMI_Pitch_X_W2 | %MD184 | Real | HMI_Tags_Warehouse_2 |  |
| HMI_Pitch_Z_W2 | %MD188 | Real | HMI_Tags_Warehouse_2 |  |
| Ist_X_W2 | %MD196 | Real | HMI_Tags_Warehouse_2 |  |
| Ist_Z_W2 | %MD200 | Real | HMI_Tags_Warehouse_2 |  |
| Soll_X_W2 | %MD204 | Real | HMI_Tags_Warehouse_2 |  |
| Soll_Z_W2 | %MD208 | Real | HMI_Tags_Warehouse_2 |  |
| Home_X_W2 | %MD212 | Real | HMI_Tags_Warehouse_2 |  |
| Home_Z_W2 | %MD216 | Real | HMI_Tags_Warehouse_2 |  |
| Ausgabe_X_W2 | %MD220 | Real | HMI_Tags_Warehouse_2 |  |
| Ausgabe_Z_W2 | %MD224 | Real | HMI_Tags_Warehouse_2 |  |
| HMI_Soll_X_W2 | %MD252 | Real | HMI_Tags_Warehouse_2 |  |
| HMI_Soll_Z_W2 | %MD256 | Real | HMI_Tags_Warehouse_2 |  |
| HMI_Jog_Step_W2 | %MD260 | Real | HMI_Tags_Warehouse_2 |  |
| Band_X_W2 | %MD264 | Real | HMI_Tags_Warehouse_2 |  |
| Band_Z_W2 | %MD268 | Real | HMI_Tags_Warehouse_2 |  |
| Band_Z_Lift_W2 | %MD272 | Real | HMI_Tags_Warehouse_2 |  |
| 5a_RFID_Code_Out | %MD300 | UDInt | 5a_RFID |  |
| HMI_Fachnummer_W2 | %MW180 | UInt | HMI_Tags_Warehouse_2 |  |
| HMI_Spalten_W2 | %MW192 | Int | HMI_Tags_Warehouse_2 |  |
| HMI_Spalten_Rechts_W2 | %MW194 | Int | HMI_Tags_Warehouse_2 |  |
| HMI_State_W2 | %MW228 | Int | HMI_Tags_Warehouse_2 |  |
| HMI_Ziel_Fach_W2 | %MW230 | Int | HMI_Tags_Warehouse_2 |  |
| HRL2_Anzahl_Belegt | %MW232 | UInt | HMI_Tags_Warehouse_2 |  |
| HRL2_Anzahl_Frei | %MW234 | UInt | HMI_Tags_Warehouse_2 |  |
| 5a_RFID_Artikelnummer_Out | %MW304 | UInt | 5a_RFID |  |
| 5a_RFID_ProductTyp_Out | %MW308 | UInt | 5a_RFID |  |
| 5a_RFID_State | %MW310 | Int | 5a_RFID |  |

### Station 5B — NW 27, 29 Warehouse_1 — 111 Tags (FIO 33 / SPS 78)

SCL: scl/Zone_5b_Hochregallager/Hochregal_Automatik_Betrieb.scl · scl/Zone_5b_Hochregallager/FB_Warehouse_Mode_Select.scl · scl/Zone_5b_Hochregallager/FB_Warehouse_Gate.scl · scl/Zone_5b_Hochregallager/FB_Warehouse_Stacker_IO.scl · scl/Zone_5b_Hochregallager/FB_Warehouse_Manual_Soll.scl · scl/Zone_5b_Hochregallager/FB_Warehouse_Actuators.scl · scl/Zone_5b_Hochregallager/FB_Einlagern.scl · scl/Zone_5b_Hochregallager/FB_Auslagern.scl · scl/Zone_5b_Hochregallager/FB_Suchen.scl · scl/Zone_5b_Hochregallager/FB_Loeschen.scl · scl/Zone_5b_Hochregallager/FB_Freies_Fach_Suchen.scl · scl/Zone_5b_Hochregallager/FB_Datenverwaltung_Lager.scl · scl/Zone_5b_Hochregallager/FB_Berechn_Offset.scl · scl/Zone_5b_Hochregallager/FB_Meldung.scl · scl/Zone_5b_Hochregallager/FB_Lagerstatus.scl · scl/Zone_5b_Hochregallager/FB_Hochregallager.scl (optional, nicht in OB1) · scl/Zone_5a_Foerderband_Lager/OB1_NW27_Belts_to_Metal_Warehouse.scl

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| 5b_Roller Conveyor (4m) 0 | %A13.0 | Bool | Forderband |  |
| 5b_Curved Roller Conveyor 2 CCW | %A13.1 | Bool | Forderband |  |
| 5b_Roller Conveyor (2m) 1 | %A13.2 | Bool | Forderband |  |
| 5b_Curved Roller Conveyor 4 | %A13.3 | Bool | Forderband |  |
| 5b_Roller Conveyor (6m) 0 | %A13.4 | Bool | Forderband |  |
| 5b_Curved Roller Conveyor 3 CCW | %A13.5 | Bool | Forderband |  |
| 5b_RFID Reader 5 Execute Command | %A13.7 | Bool | 5b_RFID |  |
| 5b_Stacker Crane 0 (Left) | %A14.0 | Bool | Warehouse |  |
| 5b_Stacker Crane 0 (Right) | %A14.1 | Bool | Warehouse |  |
| 5b_Warning_Light_5 | %A14.2 | Bool | Warehouse |  |
| 5b_Loading Conveyor 1 | %A14.3 | Bool | Warehouse |  |
| 5b_Alarm Siren 2 | %A14.4 | Bool | Warehouse |  |
| 5b_RFID Reader 5 Command | %AD124 | DInt | 5b_RFID |  |
| 5b_RFID Reader 5 Write Data | %AD128 | DInt | 5b_RFID |  |
| 5b_RFID Reader 5 Memory Index | %AD132 | DInt | 5b_RFID |  |
| 5b_Stacker Crane 0 X Set Point (V) | %AD136 | Real | Warehouse |  |
| 5b_Stacker Crane 0 Z Set Point (V) | %AD140 | Real | Warehouse |  |
| 5b_1_to_Warehouse | %E13.0 | Bool | Forderband |  |
| 5b_2_to_Warehouse | %E13.1 | Bool | Forderband |  |
| 5b_3_to_Warehouse | %E13.2 | Bool | Forderband |  |
| 5b_Pallet_vor_Regal_E13 | %E13.3 | Bool | Warehouse |  |
| 5b_Diffuse Sensor 48 | %E13.4 | Bool | Warehouse |  |
| 5b_Stacker Crane 0 Moving-X | %E13.6 | Bool | Warehouse |  |
| 5b_Stacker Crane 0 Moving-Z | %E13.7 | Bool | Warehouse |  |
| 5b_Stacker Crane 0 Left Limit | %E14.0 | Bool | Warehouse |  |
| 5b_Stacker Crane 0 Right Limit | %E14.1 | Bool | Warehouse |  |
| 5b_Stacker Crane 0 Middle Limit | %E14.2 | Bool | Warehouse |  |
| 5b_Pallet_vor_Regal | %E14.3 | Bool | Warehouse |  |
| 5b_RFID Reader 5 Status | %ED150 | DInt | 5b_RFID |  |
| 5b_RFID Reader 5 Read Data | %ED154 | DInt | 5b_RFID |  |
| 5b_RFID Reader 5 Command ID | %ED158 | DInt | 5b_RFID |  |
| 5b_Stacker Crane 0 X Position (V) | %ED162 | Real | Warehouse |  |
| 5b_Stacker Crane 0 Z Position (V) | %ED166 | Real | Warehouse |  |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| Mode_Einricht | %M60.0 | Bool | Warehouse |  |
| Mode_Auto | %M60.1 | Bool | Warehouse |  |
| Mode_Hand | %M60.2 | Bool | Warehouse |  |
| Not_Aus | %M60.3 | Bool | Warehouse |  |
| HMI_Initialisieren | %M60.4 | Bool | Warehouse |  |
| HMI_Reset_Meldung | %M60.5 | Bool | Warehouse |  |
| HMI_Pos_Speichern | %M60.6 | Bool | Warehouse |  |
| HMI_Raster_Berechnen | %M60.7 | Bool | Warehouse |  |
| HMI_Loeschen | %M61.0 | Bool | Warehouse |  |
| HMI_Suchen | %M61.1 | Bool | Warehouse |  |
| HMI_FreiesFach | %M61.2 | Bool | Warehouse |  |
| HMI_Start_Einlagern | %M61.3 | Bool | Warehouse |  |
| HMI_Start_Auslagern | %M61.4 | Bool | Warehouse |  |
| HMI_Reset | %M61.5 | Bool | Warehouse |  |
| HMI_Stop | %M61.6 | Bool | Warehouse |  |
| HMI_Copy_Ist_to_Soll | %M61.7 | Bool | Warehouse |  |
| Paket_vor_Regal | %M62.0 | Bool | Warehouse |  |
| Paket_Fuer_Hochregal | %M62.1 | Bool | Warehouse |  |
| Palette_Aufgenommen | %M62.2 | Bool | Warehouse |  |
| Palette_Abgelegt | %M62.3 | Bool | Warehouse |  |
| Position_Erreicht | %M62.4 | Bool | Warehouse |  |
| Gabel_Rechts_Ausgefahren | %M62.5 | Bool | Warehouse |  |
| Gabel_Links_Ausgefahren | %M62.6 | Bool | Warehouse |  |
| Gabel_Eingefahren | %M62.7 | Bool | Warehouse |  |
| Fahre_Zu_Position | %M63.0 | Bool | Warehouse |  |
| Palette_Aufnehmen_Cmd | %M63.1 | Bool | Warehouse |  |
| Palette_Ablegen_Cmd | %M63.2 | Bool | Warehouse |  |
| Gabel_Rechts_Cmd | %M63.3 | Bool | Warehouse |  |
| Gabel_Links_Cmd | %M63.4 | Bool | Warehouse |  |
| Gabel_Mitte_Cmd | %M63.5 | Bool | Warehouse |  |
| HRL_Busy | %M64.0 | Bool | Warehouse |  |
| HRL_Done | %M64.1 | Bool | Warehouse |  |
| HRL_Error | %M64.2 | Bool | Warehouse |  |
| HRL_Mode_Auto | %M64.3 | Bool | Warehouse |  |
| HRL_Mode_Hand | %M64.4 | Bool | Warehouse |  |
| HRL_Mode_Einricht | %M64.5 | Bool | Warehouse |  |
| 5b_RFID_Lesen | %M65.0 | Bool | 5b_RFID |  |
| RFID_5b_Busy | %M65.1 | Bool | 5b_RFID |  |
| RFID_5b_Done | %M65.2 | Bool | 5b_RFID |  |
| RFID_5b_Error | %M65.3 | Bool | 5b_RFID |  |
| RFID_5b_Bereit | %M65.4 | Bool | 5b_RFID |  |
| RFID_5b_Gueltig | %M65.5 | Bool | 5b_RFID |  |
| RFID_5b_Fehler | %M65.6 | Bool | 5b_RFID |  |
| HMI_Soll_X_Plus | %M65.7 | Bool | Warehouse |  |
| HMI_Soll_X_Minus | %M66.0 | Bool | Warehouse |  |
| HMI_Soll_Z_Plus | %M66.1 | Bool | Warehouse |  |
| HMI_Soll_Z_Minus | %M66.2 | Bool | Warehouse |  |
| RFID_5b_Materialart_Out | %MB146 | USInt | 5b_RFID |  |
| Ist_Z | %MD100 | Real | Warehouse |  |
| Soll_X | %MD104 | Real | Warehouse |  |
| Soll_Z | %MD108 | Real | Warehouse |  |
| Home_X | %MD112 | Real | Warehouse |  |
| Home_Z | %MD116 | Real | Warehouse |  |
| Ausgabe_X | %MD120 | Real | Warehouse |  |
| Ausgabe_Z | %MD124 | Real | Warehouse |  |
| RFID_5b_Code_Out | %MD140 | UDInt | 5b_RFID |  |
| HMI_Soll_X | %MD152 | Real | Warehouse |  |
| HMI_Soll_Z | %MD156 | Real | Warehouse |  |
| HMI_Jog_Step | %MD160 | Real | Warehouse |  |
| Band_X | %MD164 | Real | Warehouse |  |
| Band_Z | %MD168 | Real | Warehouse |  |
| Band_Z_Lift | %MD172 | Real | Warehouse |  |
| HMI_5b_Offset | %MD176 | Real | Warehouse |  |
| HMI_5a_Offset | %MD52 | Real | HMI_Tags_Warehouse_2 |  |
| HMI_Pitch_X | %MD84 | Real | Warehouse |  |
| HMI_Pitch_Z | %MD88 | Real | Warehouse |  |
| Ist_X | %MD96 | Real | Warehouse |  |
| HMI_State | %MW128 | Int | Warehouse |  |
| HMI_Ziel_Fach | %MW130 | Int | Warehouse |  |
| HRL_Anzahl_Belegt | %MW132 | UInt | Warehouse |  |
| HRL_Anzahl_Frei | %MW134 | UInt | Warehouse |  |
| RFID_5b_State | %MW136 | Int | 5b_RFID |  |
| RFID_5b_Status_Code | %MW138 | Int | 5b_RFID |  |
| RFID_5b_Artikelnummer_Out | %MW144 | UInt | 5b_RFID |  |
| RFID_5b_ProductTyp_Out | %MW148 | UInt | 5b_RFID |  |
| HMI_Fachnummer | %MW80 | UInt | Warehouse |  |
| HMI_Spalten | %MW92 | Int | Warehouse |  |
| HMI_Spalten_Rechts | %MW94 | Int | Warehouse |  |

### Station Standard-Variablentabelle — — — 2 Tags (FIO 1 / SPS 1)

SCL: (Querschnitt, z. B. Clock_Byte)

| Factory I/O Tag | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| Gewicht | %AD24 | DInt | Standard-Variablentabelle | System ¦ Factory I/O: Gewicht |

| SPS / PLC-Merker | Adresse | Typ | Tabelle | Kommentar |
| --- | --- | --- | --- | --- |
| Clock_Byte | %MB20 | Byte | Standard-Variablentabelle |  |

# 5  SCL-Codes (Quelldateien im Repository)

## 5.1  Vision, RFID, Handling

| SCL-Datei | Lastenheft | FUP |
| --- | --- | --- |
| Zone_2a_Foerderbaender/FB_VisionReader_Metal.scl | AF-52 | NW 5 |
| Zone_2a_Foerderbaender/OB1_NW5_Metal_Vision.scl | AF-52 | NW 5 Glue |
| Zone_2b_Vision_Foerderbaender/FB_VisionReader.scl | AF-10…13 | NW 7 |
| Zone_3a_Metall_PickPlace/PickPlace_DigitalAnalog.scl | AF-40 | NW 10 / 18 |
| Zone_4a_Metall_Palletizer_RFID/FB_GantryPickPlace.scl | AF-42 | NW 15 |
| Zone_4a_Metall_Palletizer_RFID/OB1_GantryPickPlace.scl | AF-42 | NW 15 |
| Zone_4a_Metall_Palletizer_RFID/OB1_RFID_Metal.scl | AF-24 | NW 16/21 |
| Zone_4b_Kunststoff_Palletizer_RFID/FB_Palletizer.scl | AF-43 | NW 24 |
| Zone_4b_Kunststoff_Palletizer_RFID/OB1_Palletizer.scl | AF-43 | NW 24 |
| Zone_4b_Kunststoff_Palletizer_RFID/FB_RFID_ReadWrite.scl | AF-20…25 | NW 25 / 16 |
| Zone_4b_Kunststoff_Palletizer_RFID/OB1_RFID_at_PickPlace.scl | AF-14, AF-22 | NW 25 |
| Zone_4b_Kunststoff_Palletizer_RFID/OB1_Vision_RFID_before_Pallet.scl | AF-14 | Glue 4B |
| HMI_Plant/OB1_Prod_PerDay.scl | HM-01 | Übersicht |

## 5.2  Warehouse_1 Kunststoff (FUP NW 29)

| SCL-Datei | Lastenheft | Rolle |
| --- | --- | --- |
| Hochregal_Automatik_Betrieb.scl | AF-32, 36–38 | Sequenz Einlagern |
| FB_Warehouse_Mode_Select.scl | AF-04, SI-02 | Einricht / Auto / Hand |
| FB_Warehouse_Gate.scl | AF-32a, 37 | Paket_Fuer_Hochregal |
| FB_Warehouse_Stacker_IO.scl | AF-32 | Kran Ist/Soll FIO |
| FB_Warehouse_Manual_Soll.scl | AF-35 | Einricht Jog / Band |
| FB_Warehouse_Actuators.scl | SI-01 | Leuchte / Sirene |
| FB_Einlagern.scl / FB_Auslagern.scl | AF-32 / 33 | DB belegen / freigeben |
| FB_Suchen.scl / FB_Loeschen.scl | AF-34 | RFID→Artikel→Fach / löschen |
| FB_Freies_Fach_Suchen.scl | AF-32 | nächstes freies Fach |
| FB_Datenverwaltung_Lager.scl | AF-31, HM-04 | HMI-Spiegel, Zählung |
| FB_Berechn_Offset.scl | AF-35 | Raster Fach → X/Z |
| FB_Meldung.scl | AF-05 | Info_Code 0–17 |
| FB_Lagerstatus.scl | AF-05 | Belegt/Frei-Text |
| FB_Hochregallager.scl | — | Optional Wrapper, nicht in OB1 |
| Zone_5a_Foerderband_Lager/OB1_NW27_….scl | AF-51 | FUP NW 27 Band → W2 |

## 5.3  Warehouse_2 Metall (FUP NW 28)

| SCL-Datei | Lastenheft | Rolle |
| --- | --- | --- |
| Hochregal_Automatik_Betrieb_W2.scl | AF-30a, 36 | Sequenz; State 10 Skip |
| OB1_NW28_Metal_Warehouse.scl | AF-30a | FUP-Glue NW 28 |
| OB1_RFID_5a_DB4.scl | AF-24 | Reader 0 optional |
| FB_*_W2.scl (Gate, Mode, Stacker, Manual, Actuators) | AF-04, 32a | Mirror W1 |
| FB_Einlagern_W2 / Auslagern_W2 / Suchen_W2 / Loeschen_W2 | AF-32…34 | Lagerdaten W2 |
| FB_Freies_Fach_Suchen_W2 / Datenverwaltung_W2 / Offset_W2 | AF-32, 35 | Fach / Raster |
| FB_Meldung_W2 / FB_Lagerstatus_W2 | AF-05 | Info_Code W2 |

DBs: gldb_LagerverwaltungData / _W2, gldb_AktuellerFach_HMI / _W2, gldb_Meldungen. UDTs: UDT_Fach, UDT_Lager_Raster, UDT_RFID_Product, HMI-UDTs (siehe scl/.../UDT_README.md). Nicht in OB1: Ordner legacy/ und scl/Zone3_PickPlace, scl/Zone4_Palettierer (Kopien).

# 6  Produktcodes

Artikelnummer = YY × 1000 + Materialart × 100 + Color_Code  →  Beispiel 2026 / Kunststoff / Blau = 26101

RFID_CODE = YYMMDD × 1000 + Materialart × 100 + Color_Code  →  Beispiel 20.08.2026 / Kunststoff / Blau = 260820101

| Feld | Code / Werte | SCL / Tag |
| --- | --- | --- |
| Materialart | 1 Kunststoff · 2 Metall | UDT_Fach · Vision / RFID Out |
| Color_Code | 1 Blau · 2 Grün · 3 Mixed | 2b_VisionData_Color_Code |
| ProductTyp | Lid×10 + Base (z. B. 23) | 2b_VisionData_ProductType |
| RFID Tag Index | 0 CODE · 1 Artikel · 2 Material · 3 Typ | FB_RFID_ReadWrite Job 11 |

# 7  Lager-Info_Code 0–17

Quelle: FB_Meldung / FB_Meldung_W2 → gldb_AktuellerFach_HMI(.W2).Meldung.Info_Text

| Code | Info_Text | OK |
| --- | --- | --- |
| 0 | Keine Meldung | — |
| 1 | Einlagerung erfolgreich | ja |
| 2 | Fach ist bereits belegt | nein |
| 3 | Fachnummer ungültig | nein |
| 4 | Auslagerung erfolgreich | ja |
| 5 | Fach ist bereits frei | nein |
| 6 | Datensatz gelöscht | ja |
| 7 | RFID-Code fehlt | nein |
| 8 | Kein passendes Fach gefunden | nein |
| 9 | Produkt gefunden / Gefunden ¦ Fach: … | ja |
| 10 | Lager voll | nein |
| 11 | Factory I/O Zielposition geladen | ja |
| 12 | Factory I/O Ziel erreicht | ja |
| 13 | Not-Aus aktiv | nein |
| 14 | Freies Fach gefunden | ja |
| 15 | Fach ist gesperrt | nein |
| 16 | Fachposition gespeichert | ja |
| 17 | Raster: alle Positionen berechnet | ja |

# 8  Automatik-State (HMI_State / HMI_State_W2)

W1 %MW128 · W2 %MW228 · Hochregal_Automatik_Betrieb / _W2

| State | Bedeutung (HMI-Beschriftung) | Typische Aktion |
| --- | --- | --- |
| 0 | Bereit — warte auf Palette | Gate / Produkt |
| 10 | RFID lesen… (W1 Reader 5; W2 Durchlauf) | AF-36 |
| 20 | Freies Fach suchen… | FB_Freies_Fach_Suchen |
| 30 | Fahre zum Band… | Soll = Band |
| 40–42 | Gabel Band: aus / heben / ein | Pick |
| 50 | Fahre zum Fach… | Soll = Fach X/Z |
| 55–58 | Gabel Regal: aus / senken / ein | Place |
| 60 | Lagerdaten aktualisieren… | FB_Einlagern · Info_Code 1 |
| 70 | Fahre zur Home-Position… | Home_X/Z |
| 80 | Fertig | Done-Impuls · Zähler |
| 900 | Störung — siehe Meldung | Reset |

# 9  RFID Status_Code

| Code | Bedeutung | Tag / Quittung |
| --- | --- | --- |
| 0 | OK / bereit | 2b_RFID_Status_Code %MW44 |
| 1 | Kein Tag / ungültig | Tag_Present FALSE |
| 10 | Handshake-Timeout Factory I/O | HMI Reset; Command ID muss steigen |

# 10  HMI- und Testcodes

## 10.1  HMI (HM-01…07)

| Code | Umsetzung |
| --- | --- |
| HM-01 | Übersicht · Geplante/Ist-Produktion %MD320/%MD324 · HMI_Tags_Prod_PerDay.csv |
| HM-02 | HMI_Tags_RFID_Vision.csv · Schreiben/Lesen/Prüfen/Löschen/Zurücksetzen |
| HM-03 | Setup W1/W2: Raster, Pos speichern, Soll-Jog |
| HM-04 | Operate: Suchen, Freies Fach, Löschen, Start Ein-/Auslagern (Hand) |
| HM-05 | Header Einricht/Auto/Hand, STOP, Busy/Done/Error |
| HM-06 | P&P- und Palettierer-Screens (Soll) |
| HM-07 | Lagerverwaltung Online Streamlit · nicht TP |

## 10.2  Abnahmetests T-01…T-09

| Test | Code-Bezug | Wo prüfen |
| --- | --- | --- |
| T-01 | AF-10…13 | 2b_VisionData_* Both_Ready |
| T-02 | AF-14, AF-21 | Reader 2 · Pallet_Tagged |
| T-03 | AF-32a, AF-37 | Paket_Fuer_Hochregal %M62.1 / %M72.1 |
| T-04 | AF-36, AF-38 | HMI_State 10→20→…→80 W1 |
| T-05 | AF-22, AF-36 | RFID_5b_Gueltig · Fachaktuell |
| T-06 | AF-32, AF-34 | Hand Start · Belegt · Info_Code 1 |
| T-07 | AF-35 | Info_Code 17 · Kran folgt Soll |
| T-08 | AF-25, SI-03 | Status_Code 10 → Reset → 0 |
| T-09 | AF-34a, HM-07 | Browser Suche RFID→Artikel→Fach |

# 11  Verweise

Lastenheft_Abschlussprojekt.docx — Anforderungen (Was), Version 1.6

PLC_Networks.md / Main_Program_Sweep.md — FUP-Codes NW 1–29

Station_PLC_FIO_Tags.md · PLCTags3_live.csv — 485 Tags nach Station (FIO vs SPS)

docs/03_Technik/Hardware_SPS.md · HMI_Gesamtanlage.md · Lagerverwaltung_Online.md

scl/README.md — Ordner ↔ Netzwerke

scl/Zone_5b_Hochregallager/ · Zone_5a_Metall_Hochregallager/

docs/04_Anlagenbereiche/Zone_5b_Hochregallager/INFO_CODES.md

Generator: scripts/generate_pflichtenheft.py